2D Scan Engine DE2110 Specification

*Company: DongGuan DYscan Technology Co., Ltd

*Web: www.dyscan.com

*Tel: 86-755-21032537

*Add: 21st floor, Longsheng Times Building, Industrial Road, Longhua District, Shenzhen, 518000, China

Table of Contents

Product Description	3

Features	4

Application	4

Mounting	5

Host Interface Connector	6

Cable Interface	7

Performance	8

Physical Parameter	9

Environmental	9

Accessories List	10

Depth of field	10

Product Description

DE2110 is a practical embedded barcode scanner module with leading CMOS image recognition technology and intelligent image recognition system. It can scan universal 1D 2D codes, support USB, TTL multiple interfaces, read the paper, goods, screens, other media barcodes. Easily embedded in a variety of OEM products handheld, portable, fixed and barcode collector, etc.

Features

☆ Mini size, widely used in wireless machine, self-service terminal.

☆ Read all mainstream 1D 2D barcodes in the market easily. (4mil)

☆ Support USB barcode information transmission. ( languages customization)

☆ High brightness, dark environment scanning, low power consumption.

☆ Interface: USB, TTL.

☆ Support multiple systems and languages, secondary development.

Application

| ▅ Lockers | ▅ Mobile coupons, tickets |
| --- | --- |
| ▅ Medical research | ▅ Mobile payment barcode scanning |
| ▅ Lottery machines | ▅ Self-service terminals |
| ▅ Queuing calling equipment | ▅ Self-service vending machines |
| ▅ Microcontroller development |  |

Mounting

The illustrations below show the mechanical mounting dimensions for the DE2110. The structural design should leave some space between components.

Front View（unit：mm）

Top View（unit：mm）

Host Interface Connector

The following table lists the pin functions of the ffc 12pin pitch 0.5,host interface connector on the DE2110

| PIN# | Signal | Type | Definition |
| --- | --- | --- | --- |
| 1 | - | - | N/A |
| 2 | VCC | P | Power supply 3.3 |
| 3 | GND | P | Ground |
| 4 | RXD | Input | TTL-RS232 receive |
| 5 | TXD | Output | TTL-RS232 send |
| 6 | D- | Input/Output | USB D- signal |
| 7 | D+ | Input/Output | USB D+ signal |
| 8 | - | - | N/A |
| 9 | BUZ | Output | Buzzer Output, free Low level |
| 10 | LED | Output | Decoding LED input signal, low level instruction reading codes successfully. |
| 11 | - | - | N/A |
| 12 |  | TRIG | Input | Trigger pin, keep low power level can trigger decoding. |

Cable Interface

DE2110 scanner must be connected with terminals, PC host, POS and Android host etc.

USB

TTL/RS232

Performance

| Scan Type | CMOS |
| --- | --- |
| Light Source | Red Light LED 625±10nm（aim), 5600K LEDs（ Lighting) |
| CPU | 32-bit |
| Resolution | 640*480 |
| Reading precision | ≥4mil/0.1mm(PCS90%,Code 39) |
| Decoding speed | 25CM/S |
| Depth of field | 10mm-500mm |
| Scan Mode | Manual ,Continuous, Auto Sense |
| Scan angle | Test Conditions：CODE39,10mil/0.25mm,PCS90% |
|  | Pitch:±65° |
|  | Roll: ±360° |
|  | Skew: ±60° |
| Print Contrast Signal | ≥25% |
| Ambient Light | Dark environment, indoor natural light |
| Symbologies | 1D: UPC-A，UPC-E，EAN-8，EAN-13，ISSN，ISBN，Code 128， GS1-128，ISBT 128， Code 39，Code 93，Code 32, Code 11， Interleaved 2 of 5，Matrix 2 of 5，Industrial 2 of 5，Standard 2 of 5（IATA），Codabar（NW-7），Plessey，MSI Plessey，GS1-Databar（RSS），Itf-14，etc. 2D: QR Code, Data Matrix, PDF 417, Aztec Code, Maxicode, Hanxin. etc. |

Physical Parameter

| Weight | <6g |
| --- | --- |
| Dimension | (L)16.0mm * (W)21.4mm *(H)12.0mm |
| Communication Mode | USB(USB-KBW、USB-COM），TTL |
| Interface Type | FFC 12PIN Pitch 0.5 |
| Power Supply | DC 3.3V@160mA(Max）,100ma(work) |

Environmental

| Operating temperature | -20℃ to 50℃ |
| --- | --- |
| Storage temperature | -40℃ to 70℃ |
| Humidity | 5% to 95%（non-condensing） |
| Drop Height | 1.2m |
| Temp. Test | 30 minutes for high Temp., 30 minutes for low Temp., |
| High Temp. | 60℃ |
| Low Temp. | -20℃ |
| Transport vibration Test | 10H@125RPM |

Accessories List

| Cable（Optional） | USB Cable, used to connect the DE2110 to a host device. |
| --- | --- |
|  | RS232 Cable, used to connect the DE2110 to a host device, with USB supply cable. |
| Test board（Optional） | With trigger button and buzzer, it has USB or RS232 output function. |
| Power Adaptor | Output: DC5V 1A, input: AC100~240V 50~60Hz（serial port optional). |

Depth of field

| Barcode | Precision | near | far |
| --- | --- | --- | --- |
| Code 39 | 0.1(4mil) | 40mm | 130mm |
| Code 39 | 0.127mm(5mil) | 40mm | 150mm |
| Code 39 | 0.5mm(20mil) | 50mm | 400mm |
| Code 39 | 1.0mm(40mil) | 120mm | 500mm |
| EAN-13 | 0.33mm(13.0mil) | 50mm | 250mm |
| QR Code | 20.0mil | 10mm | 210mm |
| Data Matrix | 10mil | 20mm | 70mm |
| PDF 417 | 6.8mil | 20mm | 140mm |

Test Condition：T=25℃，Illumination=200LUX，PCS=0.9