3 Color Full-color LED SMD Module

- Description：

RGB LED module is made up of a full-color LED SMD module, inputting PWM voltage through pin R, G, B to adjust the brightness of three colors (Red/blue/green) to achieve full color and mixed effect. Surely, Arduino module could makes it.

2. Specification

- Adopt 5050 full-color LED

- RGB is connected to limited current resistor to prevent from burning

- Adjust three basic colors through PWM to get different colors

- Compatible with all kinks of ports of single-chip controller

- Working Voltage：5V

- LED Drive Mode：common cathode drive

3. Arduino test code：

int redpin = 11; //select the pin for the red LED

int bluepin =10; // select the pin for the blue LED

int greenpin =9;// select the pin for the green LED

int val;

void setup() {

pinMode(redpin, OUTPUT);

pinMode(bluepin, OUTPUT);

pinMode(greenpin, OUTPUT);

Serial.begin(9600);

}

void loop()

{

for(val=255; val>0; val--)

{

analogWrite(11, val);

analogWrite(10, 255-val);

analogWrite(9, 128-val);

delay(1);

}

for(val=0; val<255; val++)

{

analogWrite(11, val);

analogWrite(10, 255-val);

analogWrite(9, 128-val);

delay(1);

}

Serial.println(val, DEC);